Steps for Success—An Initial Fitting Protocol

Product Availability:

Inspire Auto Path

- Programming cables and connectors spreadsheet is available via Inspire Online

- Auto Path protocol varies based on product family and technology level

- Select Experience Level 3 for patients experienced with amplification

- Select Experience Level 1 for patients new to amplification

QuickFit

Speech Mapping

Propel, Propel Tinnitus.

Required Inspire

- Connect hearing aids to the fitting computer via the appropriate programming accessories

- Auto Path launches automatically when hearing aids that have not been previously read are detected by Inspire

- Ensure all icons are checked and highlighted in orange

- Click Next to start the routine

- Follow all onscreen prompts

- Make sure Receiver Matrix and Acoustic Options selection in Inspire must match device configuration to accurately reflect the frequency response

- Choose the Voice Language for Indicators

- Click Next

- Select Experience Level

- Click Close to exit Auto Path and open Quick Fit

QuickFit

- Use the controls in the bottom center of the QuickFit screen to adjust for patient comments regarding output, gain and occlusion

- Click on the User Control Configuration icon to enable the user control as memory, volume control, or both. (Propel Tinnitus also offers an adjustment for the tinnitus level)

- Enable Memory 4 as the preferred automatic telephone response via the memory tabs in lower corner of the frequency response screen

Speech Mapping

- Click on Speech Mapping in the left navigation bar

- Place a check for soft (50) and loud (80) Target and Response curves in the Display menu

- Select the movie camera icon to capture a 10 second video using live or recorded voice

- Ensure the top capture is between the soft and loud curves to demonstrate audibility and comfort

Fitting Summary

FrequencyTrax in Inspire

Indicators Screen

FrequencyTrax

- Click on FrequencyTrax on the left navigation bar

- Bandwidth adjusts the Source and Target regions to maximize speech clarity

- Gain adjusts the amplification of the recreated cues to maximize audibility (ten 1dB increment steps)

- Expert Assistant accessed via the left navigation bar or QuickFit screen included additional fine tuning assistance

- Click on the Memory Tabs in the lower corner of the frequency response graphs to enable or disable FrequencyTrax per memory

Indicators

- Select Indicators from the left navigation bar

- Click on an Indicator icon to highlight it to verify audibility

- Use the Play icon to test audibility through the hearing aid

- Use the Speaker icon to demo the indicator through the computer speakers

- Use the + icons to increase or decrease presentation level as needed

- Languages may be adjusted from this screen

- Available Voice Languages may be demonstrated via the Media File Player

Fitting Summary

- Select Fitting Summary from the left navigation bar

- Hover over any screen label to highlight the hyperlink

- Click on the hyperlink to navigate to the adjustment screen if needed

- Enter detailed Fitting comments and save or print to the patient’s file if required.